
Page I of


YHC 100 Hi-hat controller instructions.

IMPORTANT : READ INSTRUCTIONS ENTIRELY BEFORE USE TO AVOID PHYSICAL DAMAGE TO YOUR CONTROLLER.

1. Attach small clamp with the hook on it to the hi-hat pedal. A. Loosen clamp screws.

B. Place clamp at the bottom of the hi-hat stand's inner rod just above the links between the pedal and the rod making sure the hook faces down and away from the foot plate.

C. Tighten screws firmly. Make sure that the clamp and hook do not obstruct movement.

Note: If you can't attach the clamp to your pedal, you may attach the silver ring on the controller's cable directly to the linkage of hi-hat stand.

2. Place bottom cymbal (the one without the electronics attached to it) on the hi-hat stand.

3. Mount hi-hat cymbals to hi-hat stand using a standard hi-hat clutch. Note that it will be necessary to adjust your clutch to accommodate the thicker top cymbal. Tighten clutch firmly.

4. Thread the female jack and cord through the hole in the bottom cymbal.

5. Adjust hi-hat cymbals to your preferred distance apart. Tighten clutch firmly.

6. Connect a stereo 1/4" cable from the female jack on the cymbal to

hi-hat trigger input.

7. Mount the controller to the hi-hat stand.

               A. Loosen clamp screws on the controller's clamp.

B. Slide the hi-hat controller's ring on to the hook of the clamp.

C. Attach the clamp to the hi-hat stand making sure the cable is directly above the hook leaving the cable loose at this time. Do not tighten controllers clamp too tight.

8. VERY IMPORTANT Adjust controller position for operation.

 A. Push down on your hi-hat stand pedal all the way closing your hi-hat cymbals to the tightest possible position.

B. Keeping your cymbals closed, loosen controllers clamp and raise it up until you feel the cable stop. Lower 1/16" and tighten.

THIS PROCESS MUST BE COMPLETED EVERY TIME YOU RE​ADJUST THE THROW OF YOUR HI-HAT CYMBALS TO AVOID OVER EXTENDING THE CABLE CAUSING DAMAGE AND TO PROVIDE MAXIMUM PERFORMANCE.

9. Connect controller to your module. Use a standard 1/4" mono cable to connect the RHC 100 controller to the hi-hat controller input on your module.

10. Begin playing and programming. It is recommended to change the trigger type to best fit your Visu-Lite cymbals. Start by closing your hi-hats and listen for the closing sound. Faster action will give you the foot splash sound. Remember that if you re-adjust your hi-hat cymbal for distance apart, you must repeat step 6 above. Repeat step #6 - 8 if you are experiencing poor or improper performance.

4/1/20W


